1. Use a screwdriver to unlock the tabs at the

bottom of the control panel. Slide the bottom

out and down.

2. Disconnect the wiring harness connectors.

3. Remove 4 Phillips screws in the door

dispenser housing.

4. Slide out the housing and disconnect the target

switch and dispenser light connectors.

5. Spread out the locking tabs and remove the

switch.

6. Push the chute duct door locking tabs back

and raise the assembly above the locking

tabs.

7. Push the lower armature locking tabs (under

the assembly) back and lift up the entire

assembly.