1. Remove the ice dispenser tray and assembly.

2. Remove 2 Phillips screws from the ice

dispenser cover.

3. Turn over the ice bucket and ice dispenser

cover. Remove the Phillips screw.

4. Remove the cover.

5. Remove the Phillips screw for the ice cube

control linkage and slide the linkage to the rear

of the ice bucket.

6. Using a pair of pliers, break the tabs off the

back cover.

7. Using 2 flat blade screwdrivers, disengage the

locking tabs at either side of the ice crusher

and remove the assembly.

8. Slide the back cover off the auger. The back

cover must be replaced because of tabs

broken off for disassembly.

9. With a flat-head screwdriver, remove the C-clip

from the end of the auger.

10. Remove the auger and ice crusher blade

assembly.